# 2. Simulation and Verification Results

The current testbench uses one DUT instance and directed self-checking tests. The following waveform captures are sufficient for a minimum functional verification report. Only the signals listed for each test need to be visible in SimVision.

## 2.1 Reset and Idle Behavior

Purpose: Confirm reset initializes the controller, interfaces, STDP outputs, and representative neuron states.

Stimulus: Hold RST_N low for several clock cycles, then release reset with no AER request applied.

Expected behavior: The FSM returns to IDLE, AER request/acknowledge outputs are inactive, STDP outputs are zero, and representative V_M entries are at RESET_VALUE.

| Signal | Expected behavior | Why it matters |
| --- | --- | --- |
| CLK | Free-running clock | Timing reference. |
| RST_N | Low during reset, then high | Shows reset interval. |
| state | IDLE after reset | Confirms controller initialization. |
| AER_IN_ACK | 0 | No input transaction is active. |
| AER_OUT_REQ | 0 | No output event is active. |
| STDP_PRE_SPIKE | 16'h0000 | No pre-spike activity. |
| STDP_POST_SPIKE | 16'h0000 | No post-spike activity. |
| V_M[0], V_M[31] | RESET_VALUE | Representative hidden/output state reset. |

## 2.2 Hidden-Only Subthreshold Update

Purpose: Verify that one incoming source updates only hidden neuron s when the hidden neuron remains below threshold.

Stimulus: Drive source s=3 with WEIGHT_IN channel 3 set to raw +32 (current Q0.7 convention).

Expected behavior: The input handshake completes, hidden neuron 3 is processed through the shared datapath, V_M[3] becomes raw 64, output neuron 19 remains unchanged, and no AER output is generated.

| Signal | Expected behavior | Why it matters |
| --- | --- | --- |
| AER_IN_REQ | Input request pulse | Marks the incoming event. |
| AER_IN_ADDR | 0x13 | Source/channel s=3. |
| AER_IN_ACK | Pulses high then returns low | Confirms input handshake. |
| state | IDLE -> WAIT_INPUT_REQ_LOW -> READ_NEURON -> APPLY_LEAK -> ADD_WEIGHT -> THRESHOLD_CHECK -> WRITE_NEURON -> IDLE | Shows normal non-spiking path. |
| neuron_id | 3 / 0_0011 | Confirms hidden neuron selection. |
| captured_weight | +32 | Confirms channel weight was latched. |
| V_M[3] | 0 -> 64 | Confirms hidden-state update. |
| V_M[19] | Remains 0 | Output layer is not touched. |
| AER_OUT_REQ | Remains 0 | No external spike below threshold. |

## 2.3 Hidden Spike Triggers Output Pass

Purpose: Demonstrate the time-multiplexed two-pass behavior when a hidden neuron fires.

Stimulus: Apply repeated +0.25-equivalent events to source 5 until hidden neuron 5 reaches threshold.

Expected behavior: Hidden neuron 5 resets after firing, then the same datapath is reused for output neuron 21 with the same captured weight. The output neuron receives one update but remains subthreshold.

| Signal | Expected behavior | Why it matters |
| --- | --- | --- |
| state | Hidden READ/LEAK/ADD/CHECK/WRITE followed by a second READ/LEAK/ADD/CHECK/WRITE | Shows one shared datapath reused twice. |
| neuron_id | 5 then 21 | Shows hidden-to-output layer selection. |
| captured_weight | Constant during both passes | Same weight is reused. |
| saved_spike | 1 after hidden threshold crossing | Triggers the output pass. |
| V_M[5] | Threshold crossing -> RESET_VALUE | Confirms hidden spike/reset. |
| V_M[21] | 0 -> 64 | Confirms output neuron update. |
| AER_OUT_REQ | Remains 0 | Hidden spike alone is internal. |

## 2.4 Output Spike, STDP Post Pulse, and AER Emission

Purpose: Verify the full event path when both the hidden and output neurons fire.

Stimulus: Continue source-5 traffic until output neuron 21 reaches threshold on its sixth output-layer update.

Expected behavior: Output neuron 21 resets, STDP_POST_SPIKE[5] pulses for one clock, AER_OUT_REQ asserts, and AER_OUT_ADDR is 0x25. The request clears after AER_OUT_ACK.

| Signal | Expected behavior | Why it matters |
| --- | --- | --- |
| state | WRITE_NEURON -> SEND_SPIKE -> WAIT_OUTPUT_ACK_LOW -> IDLE | Shows output transaction control. |
| V_M[21] | 248 -> 0 | Confirms output threshold crossing and reset. |
| STDP_POST_SPIKE | Bit 5 pulses for one CLK | Confirms post-synaptic activity indication. |
| AER_OUT_REQ | 0 -> 1 -> 0 | Shows outgoing AER request. |
| AER_OUT_ADDR | 0x25 | Confirms Team-2 tile ID and source/channel encoding. |
| AER_OUT_ACK | Asserts after REQ | Completes outgoing handshake. |

## 2.5 Captured Weight Stability

Purpose: Verify that a weight is latched at event acceptance and remains constant for both hidden and output passes.

Stimulus: Accept a source-6 event with WEIGHT_IN[6]=+32, then change the live Team-3 weight bus to -64 before processing finishes.

Expected behavior: captured_weight remains +32 throughout the event. Hidden neuron 6 fires using the captured positive weight and output neuron 22 receives the same +32 weight.

| Signal | Expected behavior | Why it matters |
| --- | --- | --- |
| WEIGHT_IN | Changes from +32 to -64 on channel 6 | Shows live input bus changes mid-event. |
| captured_weight | Remains +32 | Proves event-local weight capture. |
| neuron_id | 6 then 22 | Shows both passes use the captured value. |
| V_M[6] | Fires and resets | Hidden pass uses captured weight. |
| V_M[22] | 0 -> 64 | Output pass reuses same captured weight. |

## 2.6 Periodic Background Leak Sweep

Purpose: Verify the periodic leak scheduler decays stored membrane states without producing AER or STDP activity.

Stimulus: Prepare V_M[0] and V_M[31] to raw 64, then leave the tile idle until one background sweep occurs.

Expected behavior: With beta=0.875, both representative states decay from raw 64 to raw 56. The sweep progresses through the leak states while AER_OUT_REQ and both STDP vectors remain inactive.

| Signal | Expected behavior | Why it matters |
| --- | --- | --- |
| state | LEAK_READ -> LEAK_APPLY -> LEAK_WRITE | Shows background-leak FSM path. |
| leak_index | Advances through neuron indices | Shows the sweep traverses the state array. |
| V_M[0] | 64 -> 56 | Confirms hidden-neuron decay. |
| V_M[31] | 64 -> 56 | Confirms output-neuron decay. |
| AER_OUT_REQ | 0 | Background leak must not emit AER. |
| STDP_PRE_SPIKE | 16'h0000 | Background leak is not pre-synaptic activity. |
| STDP_POST_SPIKE | 16'h0000 | Background leak is not post-synaptic activity. |

## 2.7 Foreground Event Preempts and Resumes Background Sweep

Purpose: Verify that foreground AER traffic is not lost while a background sweep is using the shared datapath.

Stimulus: Allow the periodic sweep to begin, then assert and hold an event for source 10 until the DUT acknowledges it.

Expected behavior: The sweep yields after a safe background write, the foreground event is accepted and processed, and the sweep later resumes without restarting from neuron 0.

| Signal | Expected behavior | Why it matters |
| --- | --- | --- |
| state | Leak states -> event-processing states -> leak states | Shows arbitration between background and foreground work. |
| leak_index | Pauses and later resumes | Confirms sweep progress is preserved. |
| AER_IN_REQ | Held high while sweep is active | Models foreground arrival during maintenance. |
| AER_IN_ACK | Eventually asserts | Proves request is not lost. |
| neuron_id | 10 during foreground update | Confirms correct event target. |
| V_M[0] | Remains 56 after resumption | Shows sweep did not restart and decay neuron 0 twice. |

## 2.8 Reset Cancels Pending Background Leak State

Purpose: Verify reset clears neuron state and prevents stale periodic-leak work from writing after reset.

Stimulus: Prepare a nonzero neuron, wait until a leak request is near, assert reset, then release reset and wait less than one fresh sweep period.

Expected behavior: All neuron states return to RESET_VALUE and remain there. No AER or STDP activity occurs from stale pre-reset leak state.

| Signal | Expected behavior | Why it matters |
| --- | --- | --- |
| RST_N | Asserted low then released | Marks reset during leak scheduling. |
| state | Returns to IDLE | Confirms controller restart. |
| V_M[2] | 64 -> 0 and remains 0 | Confirms stale background write is canceled. |
| AER_OUT_REQ | 0 | No spurious event after reset. |
| STDP_PRE_SPIKE | 0 | No stale pre-spike activity. |
| STDP_POST_SPIKE | 0 | No stale post-spike activity. |

## 2.9 Verification Summary

| Verification point | Expected result | Status |
| --- | --- | --- |
| Reset / idle | Known idle state and reset membrane values | PASS |
| Hidden subthreshold update | Only hidden s updates; no output event | PASS |
| Hidden spike / output pass | Same datapath and weight reused for output 16+s | PASS |
| Output spike / AER | POST pulse and correct {0010,s} AER output | PASS |
| Captured weight | Mid-event WEIGHT_IN changes do not affect active event | PASS |
| Periodic leak | Representative hidden/output states decay by beta only | PASS |
| Sweep preemption | Foreground event accepted; sweep resumes correctly | PASS |
| Reset during leak | No stale background write after reset | PASS |